Product Specifications

AXION Pro SERIES

Indoor P1.25 Fine Pixel Pitch LED Display

• 16:9 aspect ratio

Overview

The Axion Series cabinets have a native aspect ratio of 16:9. When 16:9 cabinets are arrayed with equal cabinet height & width (i.e. 10x10 or 8x8), the entire display will retain a 16:9 viewing aspect ratio. Axion Mobile is available in pixel pitches of P1.25, P1.56, P1.875, P2.5.

Axion Mobile is front and rear serviceable. This cabinet is designed for a variety of uses including flying and ground stacking. Built with durability in mind the Axion Mobile series allows for fine pitch DVLED displays in more ruggedized applications.

Vanguard LED Displays offer a five (5) year warranty and 5% spare parts.

This product is TAA approved and is suitable for DoD, Federal, State, and Local Agency Simulation Training Simulations as well as Maritime and large aircraft trainers.

Additional Features

- IC drivers allow for customizable refresh & scan rates.

- Receiving cards allow for custom gray levels from 14-18 bit.

- Redundant power & data are available.

- No internal ribbon cables. There is a single cable-less PCB connection for data & power.

- 90 Degree angle cabinets are an available option.

- Seam brightness compensation is a standard feature.

- High gray scale & refresh rates.

- Resin encapsulated LED modules available for increased durability.

- Half size cabinets are available.

- Smart module data retention.

- Superb color & brightness uniformity.

- DC-DC Remote Power Supply available.

- Low latency available- Reduction from 4 frames to .5 frames.

- Contrast Ratio: 3000:1

Axion Technical Specifications

Axion Mobile P1.25

Specifications

INTERNAL CONNECTIONS

Data in / out		   		      RJ45

AC Power in / out		                       IEC

CABINET

Aspect Ratio				          16:9

Cabinet Material	             Die Cast Aluminum

Cabinet Size	      	      23.62” x 13.28” x 2.28”

Cabinet Resolution   	                              480 x 270

Cabinet Weight				       17 lbs.

Service Access			         	         Front

IP Rating				           IP30

ENVIRONMENTAL

Operating Temperature		      -10°C to +50°C

Storage Temperature		      -20°C to +60°C

Operating Humidity		     20% to 90% RH

AC POWER

Source Voltage		              100-240V, 50/60Hz

Max Power Consumption		         165W

Avg. Power Consumption	   	           85W

INTERNAL OPERATION

Driving Method			     1/27th Scanning

Gray Scale				   14-18 bit

IC Driver				 MBI5153

Refresh Rate				   >3840Hz

Frame Frequency			          60Hz

LED

LED Type		                    	 SMD1010

LED Configuration	                                 3 in one

Pixel Density		                           640,000/m²

Module Size		                       11.81” x 6.64”

Module Resolution	                              240 x 135

Modules Per Cabinet	                                            4

Lifetime to Half Brightness (L50)	                  100,000

Contrast Ratio				     3,000:1

Brightness			           600-800Nits

Viewing Angle			             160° x 160°

Minimum Viewing Distance		      4.2 ft.